Ping and Traceroute – Lab

This document contains my responses to the provided Ping and Traceroute lab.
The topology and addressing were supplied as part of the lab instructions.

Topology:

The topology includes:

- 2 Routers: R1-ISP and R2-Central

- 1 Server: Eagle Server

- 1 Switch: S1-Central

- 11 Pods: Each pod contains two host PCs

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1-ISP | S0/0/0 | 10.10.10.6 | 255.255.255.252 | N/A |
|  | Fa0/0 | 192.168.254.253 | 255.255.255.0 | N/A |
| R2-Central | S0/0/0 | 10.10.10.5 | 255.255.255.252 | N/A |
|  | Fa0/0 | 172.16.255.254 | 255.255.0.0 | N/A |
| Eagle Server | N/A | 192.168.254.254 | 255.255.255.0 | 192.168.254.253 |
|  | N/A | 172.31.24.254 | 255.255.255.0 | N/A |
| hostPod#A | N/A | 172.16.Pod#.1 | 255.255.0.0 | 172.16.255.254 |
| hostPod#B | N/A | 172.16.Pod#.2 | 255.255.0.0 | 172.16.255.254 |
| S1-Central | N/A | 172.16.254.1 | 255.255.0.0 | 172.16.255.254 |

Task 1: Use the ping Command to Verify Simple TCP/IP Network Connectivity.

Step 1: Verify TCP/IP Network layer connectivity on the local host computer.

Command Used: ipconfig

- Recorded information about local TCP/IP network information:

| TCP/IP Information | Value |
| --- | --- |
| IP Address | 172.16.1.2 |
| Subnet Mask | 255.255.0.0 |
| Default Gateway | 172.16.255.254 |

- Use the ping command to verify TCP/IP Network layer connectivity on the local host computer.

Command Used: ping 172.16.1.2

- Observed results of the ping  command on my computer:

| Field | Value |
| --- | --- |
| Size of packet | 32 bytes |
| Number of packets sent | 4 |
| Number of replies | 4 |
| Number of lost packets | 0 |
| Minimum delay | 1ms |
| Maximum delay | 20ms |
| Average delay | 11ms |

Step 2: Verify TCP/IP Network layer connectivity on the LAN.

Command Used: ping 172.16.255.254

- Results of the ping  command to the default Gateway:

| Field | Value |
| --- | --- |
| Size of packet | 32 bytes |
| Number of packets sent | 4 |
| Number of replies | 4 |
| Number of lost packets | 0 |
| Minimum delay | 0ms |
| Maximum delay | 41ms |
| Average delay | 10ms |

What would be the result of a loss of connectivity to the default gateway?

If the default gateway is lost:

- PCs can still communicate with each other since they're all on the same 172.16.0.0/16 network

- PCs cannot reach anything outside their network — no Eagle Server, no R1-ISP, no internet

- Basically local traffic works but all external communication fails.

Step 3: Verify TCP/IP Network layer connectivity to a remote network.

Command Used: ping 192.168.254.254

- Results of the ping  command on my computer:

| Field | Value |
| --- | --- |
| Size of packet | 32 bytes |
| Number of packets sent | 4 |
| Number of replies | 4 |
| Number of lost packets | 0 |
| Minimum delay | 6ms |
| Maximum delay | 13ms |
| Average delay | 10ms |

Task 2: Use the tracert Command to Verify TCP/IP Connectivity.

Step 1: Verify TCP/IP Network layer connectivity with the tracert command.

Run: tracert 192.168.254.254

- Record your result in the following table:

| Field | Value |
| --- | --- |
| Maximum number of hops | 30 |
| First router IP address | 172.16.255.254 |
| Second router IP address | 10.10.10.6 |
| Destination reached? | Yes |

Step 2: Observe tracert output to a host that lost network connectivity.

Run: tracert -w 5 -h 4 192.168.254.254

What would the tracert output be if R1-ISP failed?

Traceroute would stop at R2-Central and would not reach the destination.

What would the tracert output be if R2-Central failed?

Traceroute would fail at the first hop (172.16.255.254), and the destination would not be reachable.

Task 3: Challenge

Output of a ping Command using the –t Option

Output of a ping Command using the –n and –r Options